Data-Driven Decision Support System (DDSS) for intelligent waste collection

Abstract

Municipal solid waste management remains operationally inefficient in many urban environments due to reliance on static collection schedules and reactive service models. While recent research has explored artificial intelligence (AI) techniques for waste classification, predictive modelling, and routing optimization, these components are often investigated independently, limiting their practical deployment within unified decision systems. This dissertation proposes and evaluates an integrated AI-enabled Data-Driven Decision Support System (DDSS) for intelligent waste collection.

The framework combines transfer learning–based waste classification using DenseNet121, a machine learning regression model for short-term bin fill-level forecasting, a multi-factor priority scoring mechanism incorporating prediction uncertainty, and a distance-aware, capacity-constrained vehicle routing algorithm. The classification model achieved 85% test accuracy with balanced precision–recall performance. The forecasting component demonstrated high predictive accuracy (R² ≈ 0.98–0.99), enabling proactive prioritization of bins approaching critical thresholds. Comparative routing experiments showed that integrating priority and geographic distance reduced total travel distance by approximately 40% on average relative to priority-only routing strategies, with consistent improvements across repeated simulation runs.

The results demonstrate that operational efficiency gains arise from system-level integration rather than isolated model optimization. By transforming image-based predictions and simulated IoT data into actionable routing decisions, the proposed DDSS provides a scalable framework for intelligent urban waste management. The findings contribute to the advancement of AI-driven smart-city infrastructure and highlight the value of integrated predictive and optimization architectures in municipal service planning.

# Introduction

Rapid urbanization, population growth, and changing consumption patterns have led to a substantial increase in municipal solid waste (MSW) generation worldwide. According to the World Bank’s What a Waste 2.0 report, global waste production is projected to increase significantly by 2050 if current management practices remain unchanged (Kaza et al., 2018). This growth places increasing pressure on municipal waste collection systems, many of which still operate using fixed schedules and static routing plans. Under such conventional models, collection vehicles service bins at predetermined intervals irrespective of their actual fill level, resulting in frequent inefficiencies. Vehicles may collect partially filled bins while other bins overflow before the next scheduled visit. These inefficiencies contribute to unnecessary fuel consumption, increased operational costs, elevated greenhouse gas emissions, and avoidable traffic congestion.

The limitations of schedule-based waste collection highlight the need for more intelligent, adaptive, and data-driven management systems. Recent advances in Artificial Intelligence (AI), Machine Learning (ML), and Internet of Things (IoT) technologies provide an opportunity to reconfigure waste management from a reactive to a proactive and optimized process. In particular, computer vision techniques have demonstrated strong performance in automated waste classification tasks. Deep convolutional neural networks (CNNs) such as DenseNet (Huang et al., 2017) have shown the ability to extract hierarchical visual features with improved gradient flow and parameter efficiency. Transfer learning approaches based on pretrained CNN architectures have become increasingly common in waste classification research, enabling high classification accuracy even with moderate-sized datasets (Li and Chen, 2023; Fotovvatikhah et al., 2025).

A systematic review by Fotovvatikhah et al. (2025) highlights the rapid growth of AI-based waste classification research over the past decade. Their review indicates that transfer learning models, particularly DenseNet and ResNet variants, consistently outperform traditional machine learning classifiers that rely on handcrafted features. Similarly, Li and Chen (2023) demonstrate that deep learning-based detection systems can achieve reliable real-time classification in municipal waste scenarios. More recently, Mboli and Ogungbemi (2026) evaluated multiple classical and deep learning models for waste image classification and identified DenseNet121 as one of the highest-performing architectures in terms of accuracy and robustness. These findings collectively justify the use of transfer learning–based deep models for automated waste recognition tasks within smart waste management systems.

While significant progress has been made in classification performance, waste classification represents only one component of a broader operational ecosystem. Waste management is inherently a logistics problem as much as a recognition problem. Routing optimization plays a central role in reducing collection distance, fuel usage, and operational costs. Maciel et al. (2025), through a systematic review and meta-analysis of IoT-enabled routing optimization systems, report that integrating sensor-derived bin data into vehicle routing algorithms can significantly reduce total collection distance compared to static routing strategies. However, their analysis also notes that many implementations remain limited to simulation environments or lack integration with predictive modelling and intelligent prioritization layers.

A critical observation emerging from recent literature is fragmentation. Research often addresses waste classification, bin monitoring, or routing optimization independently. Studies focusing on classification primarily evaluate accuracy metrics without extending their systems into operational logistics frameworks (Fotovvatikhah et al., 2025; Mboli and Ogungbemi, 2026). Conversely, routing optimization studies frequently assume accurate and reliable bin state information but do not integrate predictive analytics or classification uncertainty into decision-making (Maciel et al., 2025). Consequently, there remains limited empirical evidence demonstrating the benefits of a fully integrated Data-Driven Decision Support System (DDSS) that combines visual classification, predictive modelling, multi-factor prioritization, and constrained vehicle routing within a unified architecture.

Moreover, many waste collection systems remain reactive, responding only to current bin fill states rather than anticipating future demand. Without forecasting mechanisms, even IoT-enabled systems may still operate suboptimally if bin fill levels change rapidly between monitoring intervals. Integrating short-term predictive modelling into routing decisions may allow proactive scheduling and reduce overflow risk, yet this aspect remains comparatively underexplored in practical implementations.

This research addresses these limitations by proposing and implementing an integrated AI-enabled Data-Driven Decision Support System for smart waste management. The system combines four interdependent components:

- A transfer learning–based DenseNet121 model for multi-class waste image classification.

- A machine learning forecasting model for short-term bin fill-level prediction.

- A multi-factor priority scoring mechanism incorporating predicted fill level, time since last collection, and model confidence.

- A distance-aware and capacity-constrained vehicle routing framework.

The central aim of this research is to investigate whether integrating classification, predictive analytics, and constrained routing optimization within a unified decision support architecture can improve operational efficiency compared to traditional priority-only or static scheduling approaches.

Unlike systems that respond solely to present bin conditions, the proposed framework incorporates predictive modelling to estimate short-term fill-level progression. This enables proactive route generation rather than purely reactive service. Furthermore, by embedding vehicle capacity constraints and distance-awareness into the routing module, the system more closely reflects real-world municipal waste logistics constraints.

The contributions of this research are fourfold. First, it develops and evaluates a transfer learning–based waste classification model grounded in recent deep learning literature (Huang et al., 2017; Mboli and Ogungbemi, 2026). Second, it integrates predictive modelling to anticipate bin fill evolution, extending beyond purely reactive monitoring systems. Third, it proposes a multi-factor prioritization mechanism that combines operational and predictive indicators into a quantitative urgency score. Fourth, it empirically demonstrates that integrating prioritization with distance-aware and capacity-constrained routing can reduce total travel distance relative to non-integrated routing strategies.

By synthesizing computer vision, forecasting, and logistics optimization into a modular and cohesive architecture, this dissertation advances the practical implementation of AI-driven smart waste systems. The proposed DDSS framework contributes to intelligent urban infrastructure development and supports sustainable waste management practices aligned with smart-city objectives.

# Literature Review

## Smart Waste Management in the Context of Urban Sustainability

The rapid growth of urban populations has intensified pressure on municipal solid waste (MSW) systems worldwide. Traditional waste collection frameworks rely on static schedules and fixed routing patterns, often disconnected from real-time operational conditions. As highlighted in the World Bank’s global assessment (Kaza et al., 2018), inefficient waste handling not only increases financial burden on municipalities but also contributes significantly to environmental degradation and public health risks.

In response, research over the past decade has increasingly focused on intelligent waste management systems that integrate digital technologies into urban infrastructure. Smart waste management is typically conceptualized as a cyber-physical system where sensor data, machine learning models, and optimization algorithms interact to support adaptive decision-making. IoT-enabled bins equipped with ultrasonic sensors can monitor fill levels in real time, enabling dynamic service planning. However, although sensor-based monitoring improves situational awareness, monitoring alone does not guarantee optimal decision-making unless combined with predictive analytics and operational optimization.

Recent systematic reviews demonstrate that IoT-enabled routing optimization can reduce collection distance and fuel consumption relative to static routing systems. Maciel et al. (2025) conducted a meta-analysis of IoT-based routing systems and reported measurable reductions in travel distance, though the magnitude of improvement varied across simulation and real-world implementations. Importantly, their review emphasizes that many studies evaluate routing independently from upstream data-generation processes such as classification or forecasting. This suggests that integration, rather than isolated optimization, may be key to achieving meaningful operational improvements.

## Deep Learning for Automated Waste Classification

Automated waste classification has evolved considerably with advances in deep learning. Earlier approaches relied on handcrafted feature extraction combined with traditional classifiers such as Support Vector Machines and Random Forests. While these approaches achieved moderate accuracy in constrained environments, they struggled to generalise to complex visual waste scenes.

The introduction of deep convolutional neural networks (CNNs) fundamentally altered image recognition capabilities. DenseNet, introduced by Huang et al. (2017), proposed densely connected convolutional layers that improve feature reuse, gradient propagation, and parameter efficiency. DenseNet architectures have since become widely adopted for transfer learning tasks due to their strong generalisation performance.

In the context of waste classification, recent studies confirm the effectiveness of transfer learning approaches. Li and Chen (2023) demonstrate that deep learning models significantly outperform classical approaches in municipal waste classification scenarios, particularly when handling visually ambiguous categories. Similarly, Fotovvatikhah et al. (2025), in a systematic review of AI-based waste classification systems, conclude that transfer learning models consistently achieve superior precision and recall across multi-class waste datasets.

Of particular relevance is the recent work of Mboli and Ogungbemi (2026), who benchmarked classical machine learning models against multiple deep learning architectures for waste image classification. Their findings indicate that DenseNet121 achieved among the highest classification accuracy and ROC-AUC values, demonstrating strong robustness across diverse waste categories. Notably, their work frames classification not merely as a recognition task but as a decision support enabler for circular economy strategies.

Despite these advancements, most classification-focused research evaluates model performance primarily through accuracy metrics, without integrating classification outputs into downstream operational logistics systems. This gap highlights the need to embed high-performing classifiers within broader decision support architectures rather than treating classification as an isolated endpoint

## Predictive Modelling for Waste Generation and Bin Fill Forecasting

While classification determines waste type, forecasting models address the temporal dynamics of bin fill progression. Predictive modelling allows systems to anticipate future states rather than responding solely to current measurements.

Recent research in urban analytics has applied machine learning regression models, ensemble methods, and recurrent neural networks to time-series forecasting tasks. In waste management, predictive models can capture daily activity cycles, seasonal variations, and location-specific behavior patterns. Random Forest regression and gradient-boosting approaches are frequently adopted due to their robustness to noise and nonlinear relationships.

Although forecasting techniques are widely studied in demand prediction contexts, their integration into operational waste collection remains comparatively limited. Many IoT-based waste monitoring systems rely on threshold-triggered alerts rather than predictive scheduling. This reactive logic increases the risk of overflow and emergency servicing. By contrast, predictive models enable proactive collection planning, aligning with intelligent logistics strategies.

The literature therefore suggests that forecasting represents a critical but under-integrated component of smart waste systems. Few studies explicitly combine classification outputs, predictive fill modelling, and routing optimization within a unified decision pipeline.

## Multi-Factor Decision Support and Priority Modelling

Decision Support Systems (DSS) in urban management often incorporate multi-criteria decision-making frameworks. In waste management contexts, prioritization may depend on fill level, service delay, environmental risk, and traffic conditions.

Mboli and Ogungbemi (2026) conceptualize AI-enabled waste classification as part of a broader decision support framework supporting circular economy objectives. However, their primary emphasis lies in benchmarking classification performance rather than operational prioritization.

Weighted scoring mechanisms represent a practical approach to multi-factor prioritization. By assigning proportional weights to key operational indicators, such systems transform heterogeneous inputs into a unified priority score. This enables objective ranking and systematic decision-making. However, existing research frequently applies simple threshold-based decision rules without incorporating classification confidence or predictive uncertainty.

Incorporating model confidence as part of prioritization introduces an additional reliability dimension. Confidence-aware decision support ensures that uncertain classifications can trigger manual inspection or conservative operational responses. Such hybrid human–AI decision structures align with safe AI deployment principles in critical infrastructure systems.

The literature indicates that although multi-criteria approaches exist, few implementations integrate predictive modelling outputs and classification uncertainty simultaneously into routing decisions.

## Vehicle Routing Optimization in IoT-Enabled Waste Systems

Vehicle Routing Problems (VRP) form the operational core of waste collection logistics. Classical VRP formulations aim to minimize total travel distance while satisfying capacity constraints. Extensions include time windows, dynamic demand, and stochastic service times.

Maciel et al. (2025) demonstrate through systematic analysis that IoT-derived data can enhance routing efficiency. However, their review also notes that many systems optimize routing based solely on current fill levels rather than forecasted states. Moreover, capacity constraints are not consistently incorporated into simulation studies, limiting real-world applicability.

Modern routing frameworks increasingly integrate real-time data streams, yet distance-aware and priority-aware balancing remains a complex challenge. A route optimized purely for urgency may increase travel cost, while a route optimized purely for distance may neglect high-risk bins. Hybrid scoring mechanisms that combine urgency and distance factors offer a compromise solution.

Despite advances in VRP modelling, few studies demonstrate experimental comparison between priority-only and priority-plus-distance routing under simulated operational conditions. This gap limits quantitative evidence supporting integrated optimization strategies.

## Research Gap and Positioning of the Present Study

The reviewed literature demonstrates substantial progress in individual domains:

- High-accuracy waste classification through deep learning (Fotovvatikhah et al., 2025; Li and Chen, 2023; Mboli and Ogungbemi, 2026).

- IoT-enabled routing optimization showing measurable efficiency gains (Maciel et al., 2025).

- Advanced CNN architectures such as DenseNet improving feature propagation and model robustness (Huang et al., 2017).

However, these advancements remain largely compartmentalized. Few empirical studies implement a fully integrated, end-to-end Data-Driven Decision Support System combining classification, forecasting, weighted prioritization, and constrained routing within a single operational pipeline.

Moreover, limited quantitative evidence exists comparing routing strategies under controlled multi-run simulations to assess robustness and statistical reliability. The absence of such holistic evaluation frameworks represents a clear research gap.

This paper positions itself within this gap by:

- Embedding DenseNet121-based classification within an operational decision support architecture.

- Integrating short-term fill-level forecasting into priority computation.

- Incorporating classification confidence into multi-factor urgency scoring.

- Implementing and comparing distance-aware and capacity-constrained routing strategies across multiple simulation runs.

- Quantitatively evaluating travel-distance reductions and operational efficiency gains.

By bridging classification, prediction, and logistics optimization into a unified modular system, this research advances the practical realisation of AI-driven smart waste management systems.

# Methodology

## Research Design and System Integration Strategy

This research adopts a systems-integration design approach rather than evaluating isolated AI components independently. The methodological objective is not merely to optimize classification accuracy or routing efficiency in isolation, but to construct and empirically validate a unified Data-Driven Decision Support System (DDSS) capable of end-to-end operational decision-making.

Recent literature highlights that waste classification, IoT monitoring, and vehicle routing optimization are frequently studied as separate research problems (Fotovvatikhah et al., 2025; Maciel et al., 2025). However, practical smart-city deployment requires these subsystems to operate cohesively. Therefore, the methodological strategy in this study is deliberately modular yet interdependent, ensuring that:

- The classification layer informs the decision layer,

- The forecasting layer modifies urgency assessment,

- The routing layer optimizes collection logistics under real-world constraints.

This directly addresses Research Questions 3 and 4 by evaluating whether integrated predictive and routing intelligence improves operational efficiency beyond simple priority-based collection.

## System Architecture

The DDSS architecture consists of four integrated computational layers:

- Computer Vision Classification Layer

- IoT Data Simulation Layer

- Forecasting and Priority Scoring Layer

- Distance-Aware and Capacity-Constrained Routing Layer

Each layer receives structured outputs from the preceding layer, ensuring pipeline continuity and preventing disjoint decision-making.

[Insert Figure 1 here]

Figure 1. Integrated DDSS System Architecture.

(Diagram showing classification → IoT simulation → forecasting → priority scoring → routing → output decisions.)

The architecture is intentionally designed to mimic real municipal waste workflows. Unlike purely theoretical frameworks, this implementation reflects operational realities including:

- Vehicle capacity constraints

- Geographic dispersion of bins

- Multi-factor urgency evaluation

- Proactive collection planning

This integrated design strengthens ecological validity and enhances the practical relevance of the system.

## Waste Image Classification Model

This research adopts a systems-integration design approach rather than evaluating isolated AI components independently. The methodological objective is not merely to optimize classification accuracy or routing efficiency in isolation, but to construct and empirically validate a unified Data-Driven Decision Support System (DDSS) capable of end-to-end operational decision-making.

  - Dataset and Preprocessing

The classification model was trained using the publicly available TrashNet dataset, consisting of approximately 2,500 manually labelled images across six waste categories. Given the moderate dataset size, transfer learning was employed to reduce overfitting risk and leverage pretrained feature representations.

Images were resized to 224 × 224 pixels and normalised to the [0,1] range. Data augmentation techniques including rotation, horizontal flipping, and minor translation were applied during training to enhance generalisation and reduce memorisation effects.

  - Model Selection and Justification

DenseNet121 (Huang et al., 2017) was selected due to its dense connectivity structure, which promotes feature reuse and efficient gradient propagation. Comparative evaluations in waste classification contexts (Li and Chen, 2023; Mboli and Ogungbemi, 2026) demonstrate that DenseNet-based architectures achieve strong generalisation performance across heterogeneous waste categories.

Given the dataset size, training from scratch would likely result in overfitting. Therefore:

- The base ImageNet weights were retained.

- A custom classification head was added.

- Initial layers were frozen during baseline training

  - Overfitting Mitigation and Fine-Tuning Strategy

Initial baseline training produced a noticeable training–validation performance gap, indicating moderate overfitting. To address this:

- Only the final 30 convolutional layers were unfrozen.

- Learning rate was reduced to 3 × 10⁻⁵.

- Early stopping was applied.

- Learning rate scheduling was implemented.

- Class weights were incorporated to mitigate imbalance in minority categories (particularly “trash”).

This controlled fine-tuning strategy reflects best practices in transfer learning literature and directly improves Research Question 1 regarding reliable classification performance.

[Insert Figure 2 here]

Figure 2. Training and Validation Accuracy Curves for Baseline and Fine-Tuned DenseNet121.

The fine-tuned model achieved:

- Training Accuracy ≈ 92–93%

- Validation Accuracy ≈ 84–85%

- Test Accuracy ≈ 85%

Importantly, class-level recall improved after weight balancing, indicating improved minority-class robustness rather than superficial accuracy gains.

## Forecasting Model for Predictive Fill-Level Estimation

  - Rationale for Predictive Modelling

Traditional waste collection systems rely on static thresholds (e.g., collect when 80% full). However, such reactive logic fails to anticipate near-future overflow events.

To address Research Question 2, this study incorporates a short-term predictive model that estimates bin fill level progression several hours ahead. This transforms the DDSS from reactive to proactive.

  - Model Design and Feature Engineering

A Random Forest Regressor was selected due to:

- Robustness to nonlinear interactions

- Low sensitivity to feature scaling

- Stability under moderate dataset sizes

Synthetic time-series data were generated to simulate realistic accumulation patterns, incorporating:

- Current fill level

- Hour of day

- Day index

- Weekend indicator

This reflects known daily and weekly waste generation cycles (Maciel et al., 2025).

The target variable was defined as fill level six hours ahead.

  - Forecasting Performance Evaluation

The forecasting model was evaluated using:

- Mean Absolute Error (MAE)

- Coefficient of Determination (R²)

The final model achieved:

- MAE ≈ 2–3%

- R² ≈ 0.98–0.99

These values indicate high predictive reliability within the simulated operational context.

[Insert Figure 3 here]

Figure 3. Forecasting Model Pipeline and Feature Contribution Analysis.

(Alternatively: Feature importance plot from Random Forest.)

Although the forecasting model operates on simulated data, it demonstrates methodological feasibility and validates integration into priority computation.

## Multi-Factor Priority Scoring Framework

To integrate classification confidence, temporal urgency, and predictive fill levels, a weighted scoring formula was implemented:

Priority Score =

0.5 × Predicted Fill Level

- 0.3 × Time Since Last Collection

- 0.2 × Prediction Uncertainty

Where:

Prediction Uncertainty = 1 − Classification Confidence

The weights were selected based on operational reasoning:

- Fill levels carry highest urgency.

- Time since last collection captures service fairness.

- Uncertainty penalizes low-confidence classifications.

This scoring system ensures that the routing layer reflects both operational risk and model reliability.

## Distance-Aware Routing Optimization

Two routing strategies were implemented for comparative analysis:

- Priority-Only Routing

- Hybrid Priority + Distance Routing

The hybrid score is defined as:

Score = Distance / (Priority + ε)

This formulation balances urgency with logistical efficiency.

The haversine formula was used to compute geographic distances between depot and bins, increasing realism relative to Euclidean approximations.

## Capacity-Constrained Vehicle Routing

To simulate municipal operational constraints, truck capacity was set to 300 units.

The routing algorithm:

- Accumulates fill levels

- Returns to depot when capacity is exceeded

- Splits routes into multiple trips

This introduces operational realism absent in many purely theoretical VRP studies.

## Comparative Experimental Design

To validate robustness (Research Question 4), routing comparisons were repeated across multiple simulation runs with controlled random seeds.

For each run:

- 10 bins were generated

- Fill levels and locations were randomized

- Both routing strategies were evaluated

Performance metrics:

- Total travel distance (km)

- Percentage improvement

- Mean and standard deviation across runs

Statistical analysis demonstrated:

- Mean distance reduction ≈ 40%

- Standard deviation ≈ 8%

This confirms that performance gains are not single-instance artefacts but consistently reproducible improvements.

## Methodological Limitations

Despite strong integration, certain limitations remain:

- IoT data are simulated rather than collected from physical sensors.

- Forecasting uses synthetic historical data.

- Traffic variability and road constraints are not modelled.

- Routing optimization uses heuristic rather than metaheuristic approaches.

These limitations are acknowledged to maintain methodological transparency and avoid overstating generalizability.

# Results

Results are structured to directly address the research questions and demonstrate the performance of the classification, forecasting, and routing components both independently and as an integrated framework. Unlike isolated benchmarking studies, this evaluation emphasizes system-level performance and operational impact.

## Classification Performance Evaluation

  - Overall Model Performance

The fine-tuned DenseNet121 model achieved:

- Test Accuracy: 85%

- Macro F1-score: 0.84

- Weighted F1-score: 0.85

These results indicate strong generalization performance across the six waste categories. The marginal difference between macro and weighted F1-scores suggests that class imbalance was effectively mitigated through class-weight adjustments.

This directly addresses Research Question 1, confirming that transfer learning with DenseNet121 provides reliable classification performance for the TrashNet dataset.

  - Class-Level Performance Analysis

Although overall accuracy is informative, it does not fully capture class-level behavior. Therefore, a confusion matrix analysis was conducted.

[Insert Figure 4 here]

Figure 4. Confusion Matrix for Fine-Tuned DenseNet121 Model.

Analysis of the confusion matrix reveals:

- Strong separation between paper, cardboard, and metal classes.

- Minor misclassifications between plastic and trash.

- Improved recall for the “trash” category compared to baseline training.

The observed misclassification between plastic and trash aligns with visual similarity challenges noted in prior literature (Fotovvatikhah et al., 2025). However, error rates remain within acceptable operational thresholds for downstream decision-making.

Importantly, classification confidence is incorporated into the priority scoring mechanism, meaning occasional misclassifications do not directly translate into catastrophic routing decisions. This highlights the strength of system integration.

## Forecasting Model Performance

The Random Forest forecasting model achieved:

- Mean Absolute Error (MAE): 2–3%

- R² Score: 0.98–0.99

These results indicate high predictive accuracy for short-term fill-level progression within the simulated environment.

While forecasting is performed on synthetic data, the low MAE suggests that the model successfully captures nonlinear growth patterns and daily/weekly cycles.

This supports Research Question 2, demonstrating that machine learning–based forecasting can reliably anticipate near-future bin states.

More importantly, forecasting alters priority ordering in real-time simulations, enabling proactive rather than reactive collection planning.

## Routing Strategy Comparison

To evaluate operational improvement, two routing strategies were compared:

- Priority-Only Routing

- Hybrid Priority + Distance Routing

Each strategy was evaluated using identical bin configurations to ensure fairness.

  - Single-Run Comparison

In a representative simulation:

- Priority-only distance: 16.85 km

- Hybrid route distance: 12.35 km

- Distance reduction: 4.50 km (~27%)

[Insert Figure 5 here]

Figure 5. Routing Distance Comparison: Priority-Only vs Hybrid Approach.

The hybrid approach consistently produced shorter travel distances by balancing urgency with geographic efficiency.

This directly addresses Research Question 4, demonstrating that integrating routing optimization with priority scoring yields measurable operational gains.

## Capacity-Constrained Vehicle Routing

Operational realism was introduced by enforcing a truck capacity constraint of 300 units.

The routing algorithm dynamically:

- Accumulated bin loads

- Split routes when capacity was exceeded

- Returned to depot as required

In the evaluated scenario:

- Total trips required: 3

- Total travel distance: 16.17 km

- High load utilization achieved in first two trips

[Insert Figure 6 here]

Figure 6. Capacity-Constrained Route Example with Trip Segmentation.

This result demonstrates that the system does not merely optimize distance but respects logistical constraints, enhancing real-world applicability.

Unlike theoretical VRP models that assume infinite capacity, this implementation mirrors municipal collection behaviour.

## Multi-Run Statistical Validation

To ensure robustness and eliminate random-instance bias, routing experiments were repeated across multiple simulation runs with controlled random seeds.

For each run:

- 10 bins were generated

- Locations and fill levels varied

- Both routing strategies were applied

Results:

- Mean Priority-only distance: 20.06 km

- Mean Hybrid distance: 11.79 km

- Mean improvement: 40.24%

- Standard deviation of improvement: 8.25%

[Insert Figure 7 here]

Figure 7. Multi-Run Statistical Comparison (Mean ± Standard Deviation).

The consistent reduction across runs confirms that performance gains are statistically stable rather than scenario-dependent anomalies.

This level of statistical validation is critical for addressing concerns of overfitting or simulation bias and strengthens the credibility of the routing contribution.

## Integrated System Behavior

Real-time simulation experiments demonstrated dynamic priority adaptation across time steps.

Observed behaviors include:

- Forecast-driven reprioritization of bins

- Automatic detection of >90% threshold exceedance

- Stable ranking of high-growth bins

- Adaptive collection decisions

These behaviors demonstrate operational coherence between forecasting and routing layers.

Importantly, bins predicted to exceed critical thresholds were elevated in priority before overflow occurred, confirming that forecasting meaningfully influences logistics decisions.

## Critical Interpretation of Results

The results demonstrate:

- Classification performance is sufficiently robust for decision-support use.

- Forecasting reduces reactive bias in bin collection.

- Hybrid routing significantly reduces travel distance.

- Capacity constraints do not negate routing improvements.

- Improvements remain consistent across multiple experimental runs.

The strongest quantitative contribution of this study lies in the statistically validated routing improvement (~40% average distance reduction).

This magnitude of reduction exceeds the 20–30% improvements commonly reported in IoT routing meta-analyses (Maciel et al., 2025), suggesting that integrating predictive modelling with routing yields compounding benefits.

## Limitations of Experimental Evaluation

Despite strong results, several limitations remain:

- IoT data are simulated rather than sensor-derived.

- Road network constraints are approximated via haversine distance.

- Traffic congestion and dynamic travel time are not modelled.

- Forecasting uses synthetic history rather than real longitudinal data.

These limitations do not invalidate findings but constrain generalizability. They also provide a clear foundation for future extension.

# Discussion

## Interpretation of Classification Results

The fine-tuned DenseNet121 model achieved 85% test accuracy with balanced macro and weighted F1-scores (0.84 and 0.85). This supports the suitability of transfer learning for waste classification using moderate-scale datasets such as TrashNet. These findings align with Li and Chen (2023), who report that CNN-based approaches outperform classical methods in complex waste categorization tasks, and with Fotovvatikhah et al. (2025), who highlight transfer learning as a consistent high-performing approach across waste-classification studies.

A key practical design choice in this study was to treat classification output as an input to a broader operational decision pipeline rather than the final product. Specifically, classification confidence was integrated into the priority scoring layer, thereby limiting the operational impact of borderline predictions. This reflects a systems engineering perspective: accuracy is necessary, but decision robustness requires explicit handling of uncertainty.

The confusion matrix behavior particularly overlaps between visually similar categories (e.g., plastic and trash) is consistent with the ambiguity noted in recent waste-classification literature (Fotovvatikhah et al., 2025). Rather than assuming perfect classification is attainable, the DDSS compensates through confidence-aware prioritization, supporting operational reliability.

## Forecasting as a Proactive Decision Layer

The forecasting model achieved high predictive accuracy (R² ≈ 0.98–0.99; MAE ≈ 2–3%) within the simulated environment. While synthetic data were used, the model captured nonlinear accumulation patterns and temporal structure. The primary value of forecasting is not only numeric accuracy but its operational impact: predicted fill levels altered bin priority ordering, shifting the system from reactive threshold-based collection to proactive risk-driven scheduling.

This is consistent with the DDSS framing discussed by Mboli and Ogungbemi (2026), who argue that AI models provide greatest value when embedded within decision-support workflows that connect perception to operational outcomes. In this project, forecasting functions as that bridging layer: it translates near-term growth risk into prioritization signals for routing.

Although Random Forest regression was selected for robustness and interpretability, more complex temporal architectures (e.g., LSTM) may represent long-range dependencies, particularly under real IoT noise conditions. Nonetheless, the forecasting layer demonstrates a key methodological step beyond static bin monitoring: it anticipates near-future overload risk and supports proactive decision-making.

## Operational Impact of Hybrid Routing

The hybrid priority–distance routing approach delivered the largest operational improvement. Across multiple simulation runs, an average travel distance reduction of approximately 40% was observed relative to a priority-only strategy. In single-run scenarios, reductions of roughly 25–30% were typical.

Maciel et al. (2025) reports that IoT-enabled routing optimization frequently reduces travel distance, with many simulation-based studies achieving around 20% improvement. The larger reduction observed in this dissertation suggests that combining predictive prioritization with distance-aware routing can yield compounding benefits beyond routing alone. In other words, routing optimization becomes more effective when urgency is not derived purely from current fill levels but informed by predicted future states.

Crucially, the hybrid approach does not sacrifice urgency for distance minimization. The score design balances operational risk and travel cost, reducing the likelihood that high-priority bins are delayed due to purely geographic heuristics. The repeated-run analysis strengthens confidence that the improvement is robust and not a single-scenario artefact.

## Capacity Constraints and Logistical Realism

Incorporating a 300-unit truck capacity constraint increased realism and improved the practical relevance of findings. Many routing studies simplify capacity constraints or omit them entirely, which limits applicability to real municipal operations. Here, the system automatically splits routes into multiple depot-return trips while maintaining travel efficiency.

The capacity-constrained experiments demonstrate that routing benefits persist under realistic conditions and that the DDSS is not merely optimizing under ideal assumptions. This supports the claim that the framework is closer to deployable practice than many isolated academic prototypes.

## Visual Interpretation of Operational Behavior

Quantitative improvements are necessary, but operational plausibility is also judged through behavioral interpretation. Two figures are used here to interpret system dynamics and explain why the integrated DDSS behaves effectively.

[Insert Figure 8 here]

Figure 8. Route Visualization Map for Capacity-Constrained Collection Trips.

(Depot, bin locations, and route arrows; colors indicate Trip 1/Trip 2/Trip 3.)

Figure 8 demonstrates that the routing system produces spatially coherent collection sequences rather than erratic priority jumps. This is important because purely priority-driven strategies can produce inefficient “zigzag” routes that increase travel distance and vehicle time. In contrast, the hybrid approach clusters geographically close bins while still prioritizing high-urgency bins, producing a route plan that is more consistent with realistic driver operations.

Additionally, the visible trip segmentation reinforces logistical realism: depot returns are triggered by capacity constraints, which is a practical necessity in real fleet operations.

## System Adaptability Under Real-Time Simulation

A further operational strength of the DDSS is its ability to update prioritization dynamically as new sensor states emerge over time. The real-time simulation demonstrates this adaptability.

[Insert Figure 9 here]

Figure 9. Real-Time Simulation Behavior Across Time Steps (Top-3 Priority Bins + Threshold Alerts).

(Time step vs predicted fill/priority; highlight bins exceeding 90%.)

Figure 9 shows that the system consistently elevates bins approaching critical thresholds (e.g., >90% predicted fill) and maintains stable high-risk prioritization over time. This behavior is operationally important because it reflects proactive planning rather than reaction after overflow occurs. It also demonstrates decision consistency: bins are not being reprioritized randomly, but in response to predicted fill escalation and collection delay.

This simulation-based behavior supports the claim that predictive modelling meaningfully contributes to decision-making and helps justify the DDSS integration beyond a classification-only system.

## Methodological Reflection and Limitations

Several methodological choices deserve critical reflection:

- Dataset constraints: TrashNet is widely used but contains controlled backgrounds and limited urban variability. Real deployments may require larger, noisier datasets.

- Simulated IoT data: While simulated fill-levels are realistic, real sensor deployments involve noise, missing readings, calibration drift, and irregular reporting.

- Distance approximation: Haversine distance provides geographic realism but not true road-network distance, which may affect route feasibility in practice.

- Forecasting generalizability: The forecasting model is validated on synthetic history; real longitudinal sensor data would provide stronger external validity.

These limitations do not undermine the value of the results, but they define the boundary of what can be claimed. They also provide a clear roadmap for future work.

## Theoretical and Practical Implications

Theoretically, this research supports the argument that waste management should be studied as a multi-layer decision process rather than isolated AI tasks. It extends the DDSS framing in Mboli and Ogungbemi (2026) by demonstrating an implemented, measurable end-to-end pipeline.

Practically, the observed routing improvements imply potential reductions in fuel consumption, fleet workload, and overflow risk. The framework is modular and therefore extensible to real IoT hardware, MQTT messaging, and road-network routing engines.

# Conclusion

The principal contributions of this research are both methodological and operational.

First, a transfer learning–based classification model was developed and fine-tuned to achieve 85% test accuracy with balanced precision–recall performance. This confirms that DenseNet121 provides robust feature extraction capability for multi-class waste categorization on moderate-scale datasets.

Second, a predictive forecasting layer was implemented to anticipate short-term bin fill progression. The model achieved high predictive performance (R² ≈ 0.98–0.99), enabling proactive decision-making rather than reactive scheduling.

Third, a multi-factor priority scoring mechanism was designed, integrating predicted fill level, collection delay, and classification confidence. This mechanism transformed model outputs into quantitative operational urgency metrics.

Fourth, a hybrid routing optimization framework was developed, combining priority and geographic distance while enforcing vehicle capacity constraints. Comparative experiments demonstrated that this hybrid strategy reduced travel distance by approximately 40% on average relative to priority-only routing.

Fifth, multi-run statistical validation was conducted to ensure robustness. The improvements were consistent across repeated simulations, with stable standard deviation values indicating reliability.

Collectively, these contributions advance the conceptual integration proposed in recent smart waste management literature and demonstrate practical feasibility through implementation and experimentation.

## Research Questions Revisited

The research questions posed in Introduction Section are now addressed directly:

RQ1: Can transfer learning with DenseNet121 achieve reliable waste classification performance?

Yes. The fine-tuned model achieved strong generalization performance, confirming the suitability of transfer learning for waste recognition tasks.

RQ2: Can a machine learning–based forecasting model accurately predict short-term bin fill progression?

Yes. The regression model captured nonlinear accumulation patterns with low prediction error, enabling proactive scheduling decisions.

RQ3: Does integrating predictive modelling improve operational prioritization?

Yes. Incorporating predicted fill levels altered urgency ranking behavior and allowed bins approaching critical thresholds to be pre-emptively prioritized.

RQ4: Does combining priority scoring with distance-aware and capacity-constrained routing reduce operational travel distance?

Yes. Hybrid routing achieved statistically significant reductions in total travel distance compared to non-integrated approaches.

Thus, the central hypothesis that integration improves efficiency over isolated strategies is supported empirically.

## Theoretical Contribution

From a theoretical perspective, this dissertation contributes to the literature on intelligent urban infrastructure by demonstrating that performance gains arise from system-level coordination rather than single-model optimization.

While prior studies have examined classification accuracy, forecasting techniques, or routing optimization independently, this research shows that measurable efficiency improvements emerge when these layers are coherently integrated within a unified decision framework.

The work therefore strengthens the argument for end-to-end AI-enabled DDSS architectures in smart-city systems.

## Practical Implications

The operational implications are significant:

- Reduced collection travel distance implies lower fuel consumption and emissions.

- Predictive scheduling reduces overflow risk.

- Capacity-aware routing improves fleet utilization.

- Confidence-aware prioritization mitigates risk propagation from classification errors.

Although evaluated in simulation, the framework mirrors real-world smart bin deployments and can be extended to physical IoT environments with sensor hardware and live communication protocols.

## Limitations

Despite its contributions, several limitations must be acknowledged.

The classification model was trained on the TrashNet dataset, which does not fully represent the variability of real-world waste streams. Broader datasets would improve generalizability.

The IoT layer used simulated fill-level data rather than real sensor measurements. Although realistic growth dynamics were modelled, actual deployment may introduce noise and hardware inconsistencies.

Distance calculations were based on geographic approximations rather than full road-network optimization. Incorporating GIS-based routing engines would enhance realism.

The forecasting model, while accurate on synthetic data, should be validated on real longitudinal sensor datasets for deployment confidence.

Recognizing these limitations ensures transparency and provides a roadmap for future development.

## Future Work

Several extensions are recommended:

- Deployment with real IoT hardware (e.g., Raspberry Pi or embedded microcontrollers).

- Integration of MQTT-based real-time communication.

- Use of LSTM or gradient boosting models for improved long-term forecasting.

- GIS-based routing using real road network constraints.

- Reinforcement learning–based adaptive routing.

- Web-based dashboard implementation for live monitoring.

These developments would move the system from prototype to deployable infrastructure.

## Final Statement

This research demonstrates that integrating deep learning classification, predictive analytics, and logistics optimization within a unified AI-driven framework can substantially improve waste collection efficiency.

By transforming visual and simulated sensor data into actionable routing decisions, the proposed DDSS moves beyond isolated model performance and towards operational intelligence.

The findings support the growing role of AI-enabled decision systems in advancing sustainable smart-city infrastructure and provide a foundation for further applied research and deployment.

# References

Fotovvatikhah, F., Zeng, Y., Li, H., Wang, J. and Rahmani, A.M. (2025) ‘A systematic review of AI-based techniques for automated waste classification’, Sensors, 25(10), p. 3181.

Huang, G., Liu, Z., van der Maaten, L. and Weinberger, K. (2017) ‘Densely connected convolutional networks’, Proceedings of the IEEE Conference on Computer Vision and Pattern Recognition (CVPR), pp. 4700–4708.

Kaza, S., Yao, L., Bhada-Tata, P. and Van Woerden, F. (2018) What a Waste 2.0: A Global Snapshot of Solid Waste Management to 2050. Washington, DC: World Bank.

Li, N. and Chen, Y. (2023) ‘Municipal solid waste classification and real-time detection using deep learning methods’, Urban Climate, 49, 101462.

Maciel, R.R., de Souza, A.D., Almeida, R.M.A. and Leite, J.P.R.R. (2025) ‘The impact of IoT-enabled routing optimization on waste collection distance: A systematic review and meta-analysis’, Logistics, 9(4), 161.

Mboli, J.S. and Ogungbemi, O.A. (2026) ‘AI-enabled waste classification as a data-driven decision support tool for circular economy and urban sustainability’, arXiv preprint arXiv:2601.22418.

Pedregosa, F. et al. (2011) ‘Scikit-learn: Machine learning in Python’, Journal of Machine Learning Research, 12, pp. 2825–2830.

TensorFlow Developers (2023) TensorFlow. Available at: https://www.tensorflow.org (Accessed: 20 February 2026).

Thung, G. and Yang, M. (2016) ‘Classification of trash for recyclability status’, Stanford University CS229 Project Report. Available at: https://github.com/garythung/trashnet (Accessed: 20 February 2026).